Learning Report –EMBEDDED

99003588

./

| Ver. Rel. No. | Release Date | Prepared. By | Reviewed By | To be approved By | Remarks/Revision Details |
| --- | --- | --- | --- | --- | --- |
| 1 |  | SANJANA GS |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

Document History

Contents

- CREATE A MAKE FILE………………………………………………………………………………………………………………………

- CREATE A STARTUP FILE…………………………………………………………………………………………………………………

- CREATE A LINKUP FILE…………………………………………………………………………………………………………………..

- DEBUGGING TECHNIQUES…………………………………………………………………………………………………………….

- BLINKING OF AN LED IN TOGGLING MODE…………………………………………………………………………………….

- BLINKING OF AN LED IN INTERNAL PUSHPULL MODE…………………………………………………………………….

- BLINKING OF AN LED IN OPEN DRIVE USING EXTERNAL PULLUP……………………………………………………..

- BLINKING OF AN LED USING EXTERNAL BUTTON……………………………………………………………………………

- GPIO INTERFACE USING HAL_DRIVERS…………………………………………………………………………………………..

- EXTERNAL INTERUPPTS USING HAL_DRIVERS……………………………………………………………………………………

- ADC USING HAL_DRIVERS……………………………………………………………………………………………………………..

- SPI USING HAL_DRIVERS……………………………………………………………………………………………………………..

- PWM USING HAL_DRIVERS………………………………………………………………………………………………………….

- USART USING HAL_DRIVERS………………………………………………………………………………………………………..

- SPI INTERFACE USING STM32F407 BOARD……………………………………………………………………………………..

//Make file

CC=arm-none-eabi-gcc

MACH=cortex-m4

CFLAGS= -c -mcpu=$(MACH) -mthumb -std=gnu11 -Wall -o0

LDFLAGS= -nostdlib -T stm.ld -Wl,-Map=final.map

all:Application.o startup.o final.elf

Application.o:Application.c

$(CC) $(CFLAGS) -o $@ $^

startup.o:startup.c

$(CC) $(CFLAGS) -o $@ $^

final.elf: Application.o startup.o

$(CC) $(LDFLAGS) -o $@ $^

clean:

rm -rf *.o *.elf

//startup file

#include<stdint.h>

#define SRAM_START  0x20000000U

#define SRAM_SIZE   (128U * 1024U) //128KB

#define SRAM_END    ((SRAM_START) + (SRAM_SIZE))

#define STACK_START   SRAM_END

extern uint32_t _etext;

extern uint32_t _sdata;

extern uint32_t _edata;

extern uint32_t _la_data;

extern uint32_t _sbss;

extern uint32_t _ebss;

//prototype of main

int main(void);

//void __libc_init_array(void);

/* function prototypes of STM32F407x system exception and IRQ handlers */

void Reset_Handler(void);

void NMI_Handler 					(void) __attribute__ ((weak, alias("Default_Handler")));

void HardFault_Handler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void MemManage_Handler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void BusFault_Handler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void UsageFault_Handler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void SVC_Handler 					(void) __attribute__ ((weak, alias("Default_Handler")));

void DebugMon_Handler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void PendSV_Handler   				(void) __attribute__ ((weak, alias("Default_Handler")));

void SysTick_Handler  				(void) __attribute__ ((weak, alias("Default_Handler")));

void WWDG_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void PVD_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void TAMP_STAMP_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void RTC_WKUP_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void RCC_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI0_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI1_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI2_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI3_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI4_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream0_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream1_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream2_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream3_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream4_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream5_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream6_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void ADC_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN1_TX_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN1_RX0_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN1_RX1_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN1_SCE_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI9_5_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM1_BRK_TIM9_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM1_UP_TIM10_IRQHandler 		(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM1_TRG_COM_TIM11_IRQHandler 	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM1_CC_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM2_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM3_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM4_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void I2C1_EV_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void I2C1_ER_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void I2C2_EV_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void I2C2_ER_IRQHandler 			(void) __attribute__ ((weak, alias("Default_Handler")));

void SPI1_IRQHandler  				(void) __attribute__ ((weak, alias("Default_Handler")));

void SPI2_IRQHandler 				(void) __attribute__ ((weak, alias("Default_Handler")));

void USART1_IRQHandler  			(void) __attribute__ ((weak, alias("Default_Handler")));

void USART2_IRQHandler  			(void) __attribute__ ((weak, alias("Default_Handler")));

void USART3_IRQHandler   			(void) __attribute__ ((weak, alias("Default_Handler")));

void EXTI15_10_IRQHandler   		(void) __attribute__ ((weak, alias("Default_Handler")));

void RTC_Alarm_IRQHandler    		(void) __attribute__ ((weak, alias("Default_Handler")));

void OTG_FS_WKUP_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM8_BRK_TIM12_IRQHandler   	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM8_UP_TIM13_IRQHandler    	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM8_TRG_COM_TIM14_IRQHandler 	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM8_CC_IRQHandler          	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA1_Stream7_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void FSMC_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void SDIO_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM5_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void SPI3_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void UART4_IRQHandler            	(void) __attribute__ ((weak, alias("Default_Handler")));

void UART5_IRQHandler            	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM6_DAC_IRQHandler         	(void) __attribute__ ((weak, alias("Default_Handler")));

void TIM7_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream0_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream1_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream2_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream3_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream4_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void ETH_IRQHandler              	(void) __attribute__ ((weak, alias("Default_Handler")));

void ETH_WKUP_IRQHandler         	(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN2_TX_IRQHandler          	(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN2_RX0_IRQHandler         	(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN2_RX1_IRQHandler         	(void) __attribute__ ((weak, alias("Default_Handler")));

void CAN2_SCE_IRQHandler         	(void) __attribute__ ((weak, alias("Default_Handler")));

void OTG_FS_IRQHandler           	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream5_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream6_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void DMA2_Stream7_IRQHandler     	(void) __attribute__ ((weak, alias("Default_Handler")));

void USART6_IRQHandler           	(void) __attribute__ ((weak, alias("Default_Handler")));

void I2C3_EV_IRQHandler          	(void) __attribute__ ((weak, alias("Default_Handler")));

void I2C3_ER_IRQHandler          	(void) __attribute__ ((weak, alias("Default_Handler")));

void OTG_HS_EP1_OUT_IRQHandler   	(void) __attribute__ ((weak, alias("Default_Handler")));

void OTG_HS_EP1_IN_IRQHandler    	(void) __attribute__ ((weak, alias("Default_Handler")));

void OTG_HS_WKUP_IRQHandler      	(void) __attribute__ ((weak, alias("Default_Handler")));

void OTG_HS_IRQHandler           	(void) __attribute__ ((weak, alias("Default_Handler")));

void DCMI_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void CRYP_IRQHandler             	(void) __attribute__ ((weak, alias("Default_Handler")));

void HASH_RNG_IRQHandler         	(void) __attribute__ ((weak, alias("Default_Handler")));

void FPU_IRQHandler              	(void) __attribute__ ((weak, alias("Default_Handler")));

uint32_t vectors[] __attribute__((section(".isr_vector")))   = {

STACK_START,

(uint32_t)Reset_Handler,

(uint32_t)NMI_Handler,

(uint32_t)HardFault_Handler,

(uint32_t)MemManage_Handler,

(uint32_t)BusFault_Handler,

(uint32_t)UsageFault_Handler,

0,

0,

0,

0,

(uint32_t)SVC_Handler,

(uint32_t)DebugMon_Handler,

0,

(uint32_t)PendSV_Handler,

(uint32_t)SysTick_Handler,

(uint32_t)WWDG_IRQHandler,

(uint32_t)PVD_IRQHandler,

(uint32_t)TAMP_STAMP_IRQHandler,

(uint32_t)RTC_WKUP_IRQHandler,

0,

(uint32_t)RCC_IRQHandler,

(uint32_t)EXTI0_IRQHandler,

(uint32_t)EXTI1_IRQHandler,

(uint32_t)EXTI2_IRQHandler,

(uint32_t)EXTI3_IRQHandler,

(uint32_t)EXTI4_IRQHandler,

(uint32_t)DMA1_Stream0_IRQHandler,

(uint32_t)DMA1_Stream1_IRQHandler,

(uint32_t)DMA1_Stream2_IRQHandler,

(uint32_t)DMA1_Stream3_IRQHandler,

(uint32_t)DMA1_Stream4_IRQHandler,

(uint32_t)DMA1_Stream5_IRQHandler,

(uint32_t)DMA1_Stream6_IRQHandler,

(uint32_t)ADC_IRQHandler,

(uint32_t)CAN1_TX_IRQHandler,

(uint32_t)CAN1_RX0_IRQHandler,

(uint32_t)CAN1_RX1_IRQHandler,

(uint32_t)CAN1_SCE_IRQHandler,

(uint32_t)EXTI9_5_IRQHandler,

(uint32_t)TIM1_BRK_TIM9_IRQHandler,

(uint32_t)TIM1_UP_TIM10_IRQHandler,

(uint32_t)TIM1_TRG_COM_TIM11_IRQHandler,

(uint32_t)TIM1_CC_IRQHandler,

(uint32_t)TIM2_IRQHandler,

(uint32_t)TIM3_IRQHandler,

(uint32_t)TIM4_IRQHandler,

(uint32_t)I2C1_EV_IRQHandler,

(uint32_t)I2C1_ER_IRQHandler,

(uint32_t)I2C2_EV_IRQHandler,

(uint32_t)I2C2_ER_IRQHandler,

(uint32_t)SPI1_IRQHandler,

(uint32_t)SPI2_IRQHandler,

(uint32_t)USART1_IRQHandler,

(uint32_t)USART2_IRQHandler,

(uint32_t)USART3_IRQHandler,

(uint32_t)EXTI15_10_IRQHandler,

(uint32_t)RTC_Alarm_IRQHandler,

(uint32_t)OTG_FS_WKUP_IRQHandler,

(uint32_t)TIM8_BRK_TIM12_IRQHandler,

(uint32_t)TIM8_UP_TIM13_IRQHandler,

(uint32_t)TIM8_TRG_COM_TIM14_IRQHandler,

(uint32_t)TIM8_CC_IRQHandler,

(uint32_t)DMA1_Stream7_IRQHandler,

(uint32_t)FSMC_IRQHandler,

(uint32_t)SDIO_IRQHandler,

(uint32_t)TIM5_IRQHandler,

(uint32_t)SPI3_IRQHandler,

(uint32_t)UART4_IRQHandler,

(uint32_t)UART5_IRQHandler,

(uint32_t)TIM6_DAC_IRQHandler,

(uint32_t)TIM7_IRQHandler,

(uint32_t)DMA2_Stream0_IRQHandler,

(uint32_t)DMA2_Stream1_IRQHandler,

(uint32_t)DMA2_Stream2_IRQHandler,

(uint32_t)DMA2_Stream3_IRQHandler,

(uint32_t)DMA2_Stream4_IRQHandler,

(uint32_t)ETH_IRQHandler,

(uint32_t)ETH_WKUP_IRQHandler,

(uint32_t)CAN2_TX_IRQHandler,

(uint32_t)CAN2_RX0_IRQHandler,

(uint32_t)CAN2_RX1_IRQHandler,

(uint32_t)CAN2_SCE_IRQHandler,

(uint32_t)OTG_FS_IRQHandler,

(uint32_t)DMA2_Stream5_IRQHandler,

(uint32_t)DMA2_Stream6_IRQHandler,

(uint32_t)DMA2_Stream7_IRQHandler,

(uint32_t)USART6_IRQHandler,

(uint32_t)I2C3_EV_IRQHandler,

(uint32_t)I2C3_ER_IRQHandler,

(uint32_t)OTG_HS_EP1_OUT_IRQHandler,

(uint32_t)OTG_HS_EP1_IN_IRQHandler,

(uint32_t)OTG_HS_WKUP_IRQHandler,

(uint32_t)OTG_HS_IRQHandler,

(uint32_t)DCMI_IRQHandler,

(uint32_t)CRYP_IRQHandler,

(uint32_t)HASH_RNG_IRQHandler,

(uint32_t)FPU_IRQHandler,

};

void Default_Handler(void)

{

while(1);

}

void Reset_Handler(void)

{

//copy .data section to SRAM

uint32_t size = (uint32_t)&_edata - (uint32_t)&_sdata;

uint8_t *pDst = (uint8_t*)&_sdata; //sram

uint8_t *pSrc = (uint8_t*)&_la_data; //flash

for(uint32_t i =0 ; i < size ; i++)

{

*pDst++ = *pSrc++;

}

//Init. the .bss section to zero in SRAM

size = (uint32_t)&_ebss - (uint32_t)&_sbss;

pDst = (uint8_t*)&_sbss;

for(uint32_t i =0 ; i < size ; i++)

{

*pDst++ = 0;

}

//__libc_init_array ();

main();

}

//link up file

#include <stdio.h>

#include<stdlib.h>

#include<string.h>

int numbers[] = { 456,345,678,567,890,456,3456,123,765,456,896,456,678,987,000,145,90};

int  someData = 90;

void array_fill_numbers(int pNumbers[], unsigned int len)

{

for ( unsigned int i = 0 ; i < len ; i++)

{

pNumbers[i] = numbers[i];//rand() % 1000;

}

}

void display_numbers(int *pNumbers, unsigned int len, char *pMessage)

{

//printf("%s",pMessage);

for (unsigned i = 0 ; i < len ; i++)

{

// printf("%5d  ",pNumbers[i]);

}

// printf("\n");

}

void swap_numbers(int *x,int *y)

{

int temp=*x;

*x=*y;

*y=temp;

#if 0

void (*jump_addr) (void);

jump_addr = (void*)0x20000009;

jump_addr();

#endif

someData = 10;

}

void bubble_sort(int *pNumbers , unsigned int len)

{

int i,j,flag=0;

for(i=0;i<len-1;i++)

{

flag=0;

for(j=0;j<len-1-i;j++)

{

if(pNumbers[j] > pNumbers[j+1])

{

swap_numbers(&pNumbers[j],&pNumbers[j+1]);

flag=1;

}

}

if(flag==0)

break;

}

}

void insertion_sort(int *pNumbers , unsigned int len)

{

int i,j,num;

for(i=1 ; i<len ; i++)

{

j=i-1;

num = pNumbers[i];

while( (j>-1) && (pNumbers[j] > num) )

{

pNumbers[j+1] = pNumbers[j];

j--;

}

pNumbers[j+1]=num;

}

}

int main()

{

unsigned int len = sizeof(numbers)/sizeof(int);

array_fill_numbers(numbers,len);

display_numbers(numbers,len,"B-unsorted array :");

bubble_sort(numbers,len);

display_numbers(numbers,len,"B-sorted array   :");

array_fill_numbers(numbers,len);

display_numbers(numbers,len,"I-unsorted array :");

insertion_sort(numbers,len);

display_numbers(numbers,len,"I-sorted array   :");

return 0;

}

//All created make file output:

//MCU specific Header file

#ifndef _STM32F4XX_H

#define _STM32F4XX_H

#include<stdint.h>

#define _vo volatile

// DEFINING MEMORY BASE ADDRESSES

#define SRAM1_BASEADDR 0x20000000U

#define SRAM2_BASEADDR 0x2001C000U

#define FLASH_BASEADDR 0x08000000U

#define ROM_BASEADDR 0x1FFF0000U

//DEFINING PHERIPHERAL BASE ADDRESSES(BUSES)

#define AHB1_BASEADDR 0x40020000U

#define AHB2_BASEADDR 0x50000000U

#define APB1_BASEADDR 0x40000000U

#define APB2_BASEADDR 0x40010000U

//DEFINING PERIPHERALS BASE ADRESSES WHICH IS HANGING ON TO AHB1 BUS //

#define GPIOA_BASEADDR (AHB1_BASEADDR + 0x0U) //0x40020000U

#define GPIOB_BASEADDR (AHB1_BASEADDR + 0x0400U) //0x40020400U

#define GPIOC_BASEADDR (AHB1_BASEADDR + 0x0800U)

#define GPIOD_BASEADDR (AHB1_BASEADDR + 0x0C00U)

#define GPIOE_BASEADDR (AHB1_BASEADDR + 0x1000U)

#define GPIOF_BASEADDR (AHB1_BASEADDR + 0x1400U)

#define GPIOG_BASEADDR (AHB1_BASEADDR + 0x1800U)

#define GPIOH_BASEADDR (AHB1_BASEADDR + 0x1C00U)

#define GPIOI_BASEADDR (AHB1_BASEADDR + 0x2000U)

#define GPIOJ_BASEADDR (AHB1_BASEADDR + 0x2400U)

#define GPIOK_BASEADDR (AHB1_BASEADDR + 0x2800U)

//AHB2

#define USB_BASEADDR (AHB2_BASEADDR + 0x03FF)

//APB1

#define TIM2_BASEADDR (APB1_BASEADDR + 0x03FF)

//I2S

#define I2S2ext_BASEADDR (APB1_BASEADDR + 0x37FF)

#define I2S3ext_BASEADDR (APB1_BASEADDR + 0x43FF)

//SPI

#define SPI2_I2S2_BASEADDR (APB1_BASEADDR + 0x3BFF)

#define SPI3_I2S3_BASEADDR (APB1_BASEADDR + 0x3FFF)

#define SPI3_I2S3_BASEADDR (APB1_BASEADDR + 0x3FFF)

//USART

#define USART2_BASEADDR (APB1_BASEADDR + 0x47FF)

#define USART3_BASEADDR (APB1_BASEADDR + 0x4BFF)

//UART

#define UART3_BASEADDR (APB1_BASEADDR + 0x4BFF)

//APB2

#define TIM1_BASEADDR (APB2_BASEADDR + 0x03FF)

//define structures

typedef struct

{

_vo uint32_t MODER;

_vo uint32_t OTYPER;

_vo uint32_t OSPEEDR;

_vo uint32_t PUPDR;

_vo uint32_t IDR;

_vo uint32_t ODR;

_vo uint32_t BSRR;

_vo uint32_t LCKR;

_vo uint32_t AFR[2];

} Gpio_RegDef_t;   // GPIO registers type//

#define GPIOA ((Gpio_RegDef_t*)GPIOA_BASEADDR)

#define GPIOB ((Gpio_RegDef_t*)GPIOB_BASEADDR)

#define GPIOC ((Gpio_RegDef_t*)GPIOC_BASEADDR)

#define GPIOD ((Gpio_RegDef_t*)GPIOD_BASEADDR)

#define GPIOE ((Gpio_RegDef_t*)GPIOE_BASEADDR)

// Gpio_RegDef_t *GPIOP[]={((Gpio_RegDef_t*)GPIOA_BASEADDR), ((Gpio_RegDef_t*)GPIOB_BASEADDR) , ((Gpio_RegDef_t*)GPIOC_BASEADDR) , ((Gpio_RegDef_t*)GPIOD_BASEADDR), ((Gpio_RegDef_t*)GPIOE_BASEADDR)};

//ENABLE CLOCK

//rcc base address

#define RCC_BASEADDR (AHB1_BASEADDR + 0x3800U)

typedef struct

{

uint32_t RCC_CR;

uint32_t PLLCFGR;

uint32_t CFGR;

uint32_t CIR;

uint32_t AHB1RSTR;

uint32_t AHB2RSTR;

uint32_t AHB3RSTR;

uint32_t Reserved0;

uint32_t APB1RSTR;

uint32_t APB2RSTR;

uint32_t Reserved1;

uint32_t Reserved2;

uint32_t AHB1ENR;

uint32_t AHB2ENR;

uint32_t AHB3ENR;

uint32_t Reserved3;

uint32_t APB1ENR;

uint32_t APB2ENR;

uint32_t Reserved4;

uint32_t Reserved5;

uint32_t AHB1LPENR;

uint32_t AHB2LPENR;

uint32_t AHB3LPENR;

uint32_t Reserved6;

uint32_t APB1LPENR;

uint32_t APB2LPENR;

uint32_t Reserved7;

uint32_t Reserved8;

uint32_t BDCR;

uint32_t CSR;

uint32_t Reserved9;

uint32_t Reserved10;

uint32_t SSCGR;

uint32_t PLLI2SCFGR;

}RCC_RegDef_t;

#define RCC ((RCC_RegDef_t*)RCC_BASEADDR)

//PHERIFERAL CLOCK ENABLE

#define GPIOA_PCLK_EN()       (RCC -> AHB1ENR |= (1<<0))

#define GPIOB_PCLK_EN()       (RCC -> AHB1ENR |= (1<<1))

#define GPIOC_PCLK_EN()       (RCC -> AHB1ENR |= (1<<2))

#define GPIOD_PCLK_EN()       (RCC -> AHB1ENR |= (1<<3))

#define GPIOE_PCLK_EN()       (RCC -> AHB1ENR |= (1<<4))

//PHERIFERAL CLOCK DISABLE

//TODO

#define GPIOA_PCLK_DS()       (RCC -> AHB1RSTR &= ~(1<<0)) //todo

#define GPIOB_PCLK_DS()       (RCC -> AHB1RSTR &= ~(1<<1))

#define GPIOC_PCLK_DS()       (RCC -> AHB1RSTR &= ~(1<<2))

#define GPIOD_PCLK_DS()       (RCC -> AHB1RSTR &= ~(1<<3))

#define GPIOE_PCLK_DS()       (RCC -> AHB1RSTR &= ~(1<<4))

// SOME GENERIC MACROS

#define ENABLE    1

#define DISABLE   0

#define SET       ENABLE

#define RESET     DISABLE

#define GPIO_PIN_SET ENABLE

#define GPIO_PIN_RESET DISABLE

//RCC RESET PHERIFERALS

#define GPIOA_REG_RESET()

do

{ (RCC->AHB1RSTR |= (1<<0)); (RCC->AHB1RSTR &= ~(1 << 0));

} while(0)

#define GPIOB_REG_RESET()

do{ (RCC->AHB1RSTR |= (1<<1)); (RCC->AHB1RSTR &= ~(1 << 1));

} while(0)

#define GPIOC_REG_RESET()

do{ (RCC->AHB1RSTR |= (1<<2)); (RCC->AHB1RSTR &= ~(1 << 2));

} while(0)

#define GPIOD_REG_RESET()

do{ (RCC->AHB1RSTR |= (1<<3)); (RCC->AHB1RSTR &= ~(1 << 3));

} while(0)

#define GPIOE_REG_RESET()

do{ (RCC->AHB1RSTR |= (1<<4)); (RCC->AHB1RSTR &= ~(1 << 4));

} while(0)

#endif

Drivers specific header file:

#ifndef _GPIO_DRIVER_H

#define _GPIO_DRIVER_H

#include "stm32f4xx.h"

typedef struct

{

uint8_t GPIO_PinNumber;

uint8_t GPIO_PinMode;

uint8_t GPIO_PinSpeed;

uint8_t GPIO_PinPuPdControl;

uint8_t GPIO_PinOPType;

uint8_t GPIO_PinAltFuncMode;

} GPIO_Pinconfig_t;

typedef struct

{

Gpio_RegDef_t *pGPIOx;

GPIO_Pinconfig_t GPIO_PinConfig;

}GPIO_Handle_t;

// PIN NUMBERS

#define GPIO_PIN_NO_O   0

#define GPIO_PIN_NO_1   1

#define GPIO_PIN_NO_2   2

#define GPIO_PIN_NO_3   3

#define GPIO_PIN_NO_4   4

#define GPIO_PIN_NO_5   5

#define GPIO_PIN_NO_6   6

#define GPIO_PIN_NO_7   7

#define GPIO_PIN_NO_8   8

#define GPIO_PIN_NO_9   9

#define GPIO_PIN_NO_10  10

#define GPIO_PIN_NO_11  11

#define GPIO_PIN_NO_12  12

#define GPIO_PIN_NO_13  13

#define GPIO_PIN_NO_14  14

#define GPIO_PIN_NO_15  15

//MODES OF GPIO'S

#define GPIO_MODE_IN         0

#define GPIO_MODE_OUT        1

#define GPIO_MODE_ALT_FUNC   2

#define GPIO_MODE_ANALOG     3

//OUTPUT TYPE

#define GPIO_OP_TYPE_PP   0

#define GPIO_OP_TYPE_OD   1

//SPEED OG GPIO

#define GPIO_SPEED_LOW         0

#define GPIO_SPEED_MEDIUM      1

#define GPIO_SPEED_HIGH        2

#define GPIO_SPEED_VERY_HIGH   3

//GPIO PIN FOR PULL UP AND PULL DOWN

#define GPIO_N0_PUPD      0

#define GPIO_PU        1

#define GPIO_PD        2

/*

* To control clock setting

*/

void GPIO_PeriClockControl(Gpio_RegDef_t *pGPIOx, uint8_t EnorDi);

/*

*  To do initialization

*/

void GPIO_Init(GPIO_Handle_t *pGPIOHandle);

void GPIO_DeInit(Gpio_RegDef_t *pGPIOx);

/*

* read from input pin

*/

uint8_t GPIO_ReadFromInputPin(Gpio_RegDef_t *pGPIOx, uint8_t PinNumber);

/*

* read from input port

*/

uint16_t GPIO_ReadFromInputPort(Gpio_RegDef_t *pGPIOx);

/*

* write to output pin

*/

void GPIO_WriteToOutputPin(Gpio_RegDef_t *pGPIOx,uint8_t PinNumber,uint8_t value);

/*

* read from output port

*/

void GPIO_WriteToOutputPort(Gpio_RegDef_t *pGPIOx,uint16_t value);

/*

* to do

*/

void GPIO_ToggleOutputPin(Gpio_RegDef_t *pGPIOx,uint8_t PinNumber);

#endif

Driver’s source file:

#include "stm32f4xx.h"

#include "gpio_driver.h"

void GPIO_PeriClockControl(Gpio_RegDef_t *pGPIOx, uint8_t EnorDi)

{

RCC_RegDef_t *rcc_ahd1en = &(RCC->AHB1ENR);

if(EnorDi== ENABLE)

{

if(pGPIOx == GPIOA)

{

GPIOA_PCLK_EN();

}

else if(pGPIOx == GPIOB)

{

GPIOB_PCLK_EN();

}

else if(pGPIOx == GPIOC)

{

GPIOC_PCLK_EN();

}

else if(pGPIOx == GPIOD)

{

GPIOD_PCLK_EN();

}

else if(pGPIOx == GPIOE)

{

GPIOE_PCLK_EN();

}

}

else

{

if(pGPIOx == GPIOA)

{

GPIOA_PCLK_DS();

}

else if(pGPIOx == GPIOB)

{

GPIOB_PCLK_DS();

}

else if(pGPIOx == GPIOC)

{

GPIOC_PCLK_DS();

}

else if(pGPIOx == GPIOD)

{

GPIOD_PCLK_DS();

}

else if(pGPIOx == GPIOE)

{

GPIOE_PCLK_DS();

}

}

}

/*

*  To do

*/

void GPIO_Init(GPIO_Handle_t *pGPIOHandle)

{

//1. CONFIGURATION OF THE MODE OF GPIOX

uint32_t temp=0;

temp=(pGPIOHandle->GPIO_PinConfig.GPIO_PinMode<<           (2*pGPIOHandle>GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->MODER&=~(0x3<<(2*pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->MODER |=temp; //set the value

//2. CONIGURE THE SPEED

temp=( pGPIOHandle->GPIO_PinConfig.GPIO_PinSpeed<< (2*pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->OSPEEDR &=~(0x3<<(2*pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->OSPEEDR |=temp;

//3. PULL UP AND PULL DOWN REGISTER

temp=(pGPIOHandle->GPIO_PinConfig.GPIO_PinPuPdControl<< (2*pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->PUPDR &=~(0x3<<(2*pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->PUPDR |=temp;

//4. OUTPUT TYPE REGISTER

temp=( pGPIOHandle->GPIO_PinConfig.GPIO_PinOPType << (pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->OTYPER&=~(0x1<<(pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber));

pGPIOHandle->pGPIOx->OTYPER |=temp;

//5. ALTERNATING FUNCTION LOW REGISTER

if(pGPIOHandle->GPIO_PinConfig.GPIO_PinMode==   GPIO_MODE_ALT_FUNC)

{

uint8_t temp1,temp2;

temp1=(pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber)/8;

temp2=(pGPIOHandle->GPIO_PinConfig.GPIO_PinNumber)%8;

pGPIOHandle->pGPIOx->AFR[temp1] &= ~(0xf<<(4*temp2));

pGPIOHandle->pGPIOx->AFR[temp1] |= pGPIOHandle->GPIO_PinConfig.GPIO_PinAltFuncMode << (4*temp2);

}

void GPIO_DeInit(Gpio_RegDef_t *pGPIOx)

{

if(pGPIOx == GPIOA)

{

GPIOA_REG_RESET();

}else if (pGPIOx == GPIOB)

{

GPIOB_REG_RESET();

}else if (pGPIOx == GPIOC)

{

GPIOC_REG_RESET();

}else if (pGPIOx == GPIOD)

{

GPIOD_REG_RESET();

}else if (pGPIOx == GPIOE)

{

GPIOE_REG_RESET();

}

}

/*

*

*/

uint8_t GPIO_ReadFromInputPin(Gpio_RegDef_t *pGPIOx, uint8_t PinNumber)

{

uint8_t value;

//value=(uint8_t)pGPIOx->IDR;

value=(uint8_t) ((pGPIOx->IDR >> PinNumber)& 0x00000001); //shift the the value to 0th bit and and it with 1 .so we can get the value

return(value);

}

uint16_t GPIO_ReadFromInputPort(Gpio_RegDef_t *pGPIOx)

{

uint16_t value;

value = pGPIOx->IDR;

return(value);

}

/*

*

*/

void GPIO_WriteToOutputPin(Gpio_RegDef_t *pGPIOx,uint8_t PinNumber,uint8_t value)

{

if(value == GPIO_PIN_SET)

{

pGPIOx->ODR |= (1<<PinNumber);

}

else

{

pGPIOx->ODR &= ~(1<<PinNumber);

}

}

void GPIO_WriteToOutputPort(Gpio_RegDef_t *pGPIOx,uint16_t value)

{

}

/*

* TO DO

*/

void GPIO_ToggleOutputPin(Gpio_RegDef_t *pGPIOx,uint8_t PinNumber)

{

pGPIOx->ODR ^= (1<<PinNumber);

}

LED TOGGLE

MAIN FILE:

#include "stm32f407xx.h"

void delay(void)

{

for(uint32_t i = 0 ; i < 500000 ; i ++);

}

int main(void)

{

GPIO_Handle_t GpioLed;

GpioLed.pGPIOx = GPIOD;

GpioLed.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

GpioLed.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

GpioLed.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GpioLed.GPIO_PinConfig.GPIO_PinOPType = GPIO_OP_TYPE_OD;

GpioLed.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

GPIO_PeriClockControl(GPIOD,ENABLE);

GPIO_Init(&GpioLed);

while(1)

{

GPIO_ToggleOutputPin(GPIOD,GPIO_PIN_NO_12);

delay();

}

return 0;

}

LED BUTTON:

#include "stm32f407xx.h"

#define HIGH 1

#define BTN_PRESSED HIGH

void delay(void)

{

for(uint32_t i = 0 ; i < 500000/2 ; i ++);

}

int main(void)

{

GPIO_Handle_t GpioLed, GPIOBtn;

//this is led gpio configuration

GpioLed.pGPIOx = GPIOD;

GpioLed.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

GpioLed.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

GpioLed.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GpioLed.GPIO_PinConfig.GPIO_PinOPType = GPIO_OP_TYPE_PP;

GpioLed.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

GPIO_PeriClockControl(GPIOD,ENABLE);

GPIO_Init(&GpioLed);

//this is btn gpio configuration

GPIOBtn.pGPIOx = GPIOA;

GPIOBtn.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_0;

GPIOBtn.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_IN;

GPIOBtn.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GPIOBtn.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

GPIO_PeriClockControl(GPIOA,ENABLE);

GPIO_Init(&GPIOBtn);

while(1)

{

if(GPIO_ReadFromInputPin(GPIOA,GPIO_PIN_NO_0) ==      BTN_PRESSED)

{

//delay();

GPIO_ToggleOutputPin(GPIOD,GPIO_PIN_NO_12);

}

}

return 0;

}

LED WITH  EXTERNAL BUTTON:

#include "stm32f407xx.h"

#define HIGH 1

#define LOW 0

#define BTN_PRESSED LOW

void delay(void)

{

for(uint32_t i = 0 ; i < 500000/2 ; i ++);

}

int main(void)

{

GPIO_Handle_t GpioLed, GPIOBtn;

//this is led gpio configuration

GpioLed.pGPIOx = GPIOA;

GpioLed.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_8;

GpioLed.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

GpioLed.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GpioLed.GPIO_PinConfig.GPIO_PinOPType = GPIO_OP_TYPE_PP;

GpioLed.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

GPIO_PeriClockControl(GPIOA,ENABLE);

GPIO_Init(&GpioLed);

//this is btn gpio configuration

GPIOBtn.pGPIOx = GPIOB;

GPIOBtn.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

GPIOBtn.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_IN;

GPIOBtn.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GPIOBtn.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_PIN_PU;

GPIO_PeriClockControl(GPIOB,ENABLE);

GPIO_Init(&GPIOBtn);

while(1)

{

if(GPIO_ReadFromInputPin(GPIOB,GPIO_PIN_NO_12) == BTN_PRESSED)

{

delay();

GPIO_ToggleOutputPin(GPIOA,GPIO_PIN_NO_8);

}

}

return 0;

}

STM IDE TASKS:

- GPIO INTERFACE USING HAL_LIBRARY

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* <h2><center>&copy; Copyright (c) 2021 STMicroelectronics.

* All rights reserved.</center></h2>

*

* This software component is licensed by ST under BSD 3-Clause license,

* the "License"; You may not use this file except in compliance with the

* License. You may obtain a copy of the License at:

*                        opensource.org/licenses/BSD-3-Clause

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

/* USER CODE BEGIN 2 */

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

HAL_GPIO_TogglePin(GREEN_LED_GPIO_Port, GREEN_LED_Pin);

HAL_Delay(500);

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GREEN_LED_GPIO_Port, GREEN_LED_Pin, GPIO_PIN_SET);

/*Configure GPIO pin : GREEN_LED_Pin */

GPIO_InitStruct.Pin = GREEN_LED_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_HIGH;

HAL_GPIO_Init(GREEN_LED_GPIO_Port, &GPIO_InitStruct);

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/

- EXTERNAL INTERUPPT USING HAL_LIBRARY

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* <h2><center>&copy; Copyright (c) 2021 STMicroelectronics.

* All rights reserved.</center></h2>

*

* This software component is licensed by ST under BSD 3-Clause license,

* the "License"; You may not use this file except in compliance with the

* License. You may obtain a copy of the License at:

*                        opensource.org/licenses/BSD-3-Clause

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN PV */

uint8_t flag=0;

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

/* USER CODE BEGIN 2 */

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

if(flag==1)

{

HAL_GPIO_TogglePin(GREEN_LED_GPIO_Port,GREEN_LED_Pin);

flag=0;

}

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GREEN_LED_GPIO_Port, GREEN_LED_Pin, GPIO_PIN_RESET);

/*Configure GPIO pin : BLUE_BUTTON_Pin */

GPIO_InitStruct.Pin = BLUE_BUTTON_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_FALLING;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(BLUE_BUTTON_GPIO_Port, &GPIO_InitStruct);

/*Configure GPIO pin : GREEN_LED_Pin */

GPIO_InitStruct.Pin = GREEN_LED_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GREEN_LED_GPIO_Port, &GPIO_InitStruct);

/* EXTI interrupt init*/

HAL_NVIC_SetPriority(EXTI15_10_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI15_10_IRQn);

}

/* USER CODE BEGIN 4 */

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

flag=1;

}

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/

- ADC  USING HAL_LIBRARY

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* <h2><center>&copy; Copyright (c) 2021 STMicroelectronics.

* All rights reserved.</center></h2>

*

* This software component is licensed by ST under BSD 3-Clause license,

* the "License"; You may not use this file except in compliance with the

* License. You may obtain a copy of the License at:

*                        opensource.org/licenses/BSD-3-Clause

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

ADC_HandleTypeDef hadc1;

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_ADC1_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

uint32_t adcVal0,adcVal1;

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

ADC_ChannelConfTypeDef sConfig = {0};

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_ADC1_Init();

/* USER CODE BEGIN 2 */

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

sConfig.Channel=ADC_CHANNEL_0;

HAL_ADC_ConfigChannel(&hadc1, &sConfig);

HAL_ADC_Start(&hadc1);

if( HAL_ADC_PollForConversion(&hadc1,5)==HAL_OK)

{

adcVal0=HAL_ADC_GetValue(&hadc1);

}

sConfig.Channel=ADC_CHANNEL_1;

HAL_ADC_ConfigChannel(&hadc1, &sConfig);

HAL_ADC_Start(&hadc1);

if( HAL_ADC_PollForConversion(&hadc1,5)==HAL_OK)

{

adcVal1=HAL_ADC_GetValue(&hadc1);

}

HAL_Delay(50);

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief ADC1 Initialization Function

* @param None

* @retval None

*/

static void MX_ADC1_Init(void)

{

/* USER CODE BEGIN ADC1_Init 0 */

/* USER CODE END ADC1_Init 0 */

/* USER CODE BEGIN ADC1_Init 1 */

/* USER CODE END ADC1_Init 1 */

/** Configure the global features of the ADC (Clock, Resolution, Data Alignment and number of conversion)

*/

hadc1.Instance = ADC1;

hadc1.Init.ClockPrescaler = ADC_CLOCK_SYNC_PCLK_DIV2;

hadc1.Init.Resolution = ADC_RESOLUTION_8B;

hadc1.Init.ScanConvMode = DISABLE;

hadc1.Init.ContinuousConvMode = DISABLE;

hadc1.Init.DiscontinuousConvMode = DISABLE;

hadc1.Init.ExternalTrigConvEdge = ADC_EXTERNALTRIGCONVEDGE_NONE;

hadc1.Init.ExternalTrigConv = ADC_SOFTWARE_START;

hadc1.Init.DataAlign = ADC_DATAALIGN_RIGHT;

hadc1.Init.NbrOfConversion = 1;

hadc1.Init.DMAContinuousRequests = DISABLE;

hadc1.Init.EOCSelection = ADC_EOC_SINGLE_CONV;

if (HAL_ADC_Init(&hadc1) != HAL_OK)

{

Error_Handler();

}

/** Configure for the selected ADC regular channel its corresponding rank in the sequencer and its sample time.

*/

sConfig.Channel = ADC_CHANNEL_0;

sConfig.Rank = 1;

sConfig.SamplingTime = ADC_SAMPLETIME_28CYCLES;

if (HAL_ADC_ConfigChannel(&hadc1, &sConfig) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN ADC1_Init 2 */

/* USER CODE END ADC1_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_RESET);

/*Configure GPIO pin : PC13 */

GPIO_InitStruct.Pin = GPIO_PIN_13;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pin : PA5 */

GPIO_InitStruct.Pin = GPIO_PIN_5;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/

- SPI USING HAL_LIBRARY

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* <h2><center>&copy; Copyright (c) 2021 STMicroelectronics.

* All rights reserved.</center></h2>

*

* This software component is licensed by ST under BSD 3-Clause license,

* the "License"; You may not use this file except in compliance with the

* License. You may obtain a copy of the License at:

*                        opensource.org/licenses/BSD-3-Clause

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

SPI_HandleTypeDef hspi1;

/* USER CODE BEGIN PV */

uint8_t buffer_tx[10]={30,31,32,33,34,35,36,37,38,39};

uint8_t buffer_rx[10];

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_SPI1_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_SPI1_Init();

/* USER CODE BEGIN 2 */

HAL_SPI_TransmitReceive(&hspi1,buffer_tx,buffer_rx,10,100);

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief SPI1 Initialization Function

* @param None

* @retval None

*/

static void MX_SPI1_Init(void)

{

/* USER CODE BEGIN SPI1_Init 0 */

/* USER CODE END SPI1_Init 0 */

/* USER CODE BEGIN SPI1_Init 1 */

/* USER CODE END SPI1_Init 1 */

/* SPI1 parameter configuration*/

hspi1.Instance = SPI1;

hspi1.Init.Mode = SPI_MODE_MASTER;

hspi1.Init.Direction = SPI_DIRECTION_2LINES;

hspi1.Init.DataSize = SPI_DATASIZE_8BIT;

hspi1.Init.CLKPolarity = SPI_POLARITY_LOW;

hspi1.Init.CLKPhase = SPI_PHASE_1EDGE;

hspi1.Init.NSS = SPI_NSS_SOFT;

hspi1.Init.BaudRatePrescaler = SPI_BAUDRATEPRESCALER_8;

hspi1.Init.FirstBit = SPI_FIRSTBIT_MSB;

hspi1.Init.TIMode = SPI_TIMODE_DISABLE;

hspi1.Init.CRCCalculation = SPI_CRCCALCULATION_DISABLE;

hspi1.Init.CRCPolynomial = 10;

if (HAL_SPI_Init(&hspi1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN SPI1_Init 2 */

/* USER CODE END SPI1_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOA_CLK_ENABLE();

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/

- PWM USING HAL_LIBRARY

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* <h2><center>&copy; Copyright (c) 2021 STMicroelectronics.

* All rights reserved.</center></h2>

*

* This software component is licensed by ST under BSD 3-Clause license,

* the "License"; You may not use this file except in compliance with the

* License. You may obtain a copy of the License at:

*                        opensource.org/licenses/BSD-3-Clause

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

TIM_HandleTypeDef htim2;

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_TIM2_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_TIM2_Init();

/* USER CODE BEGIN 2 */

HAL_TIM_PWM_Start(&htim2, TIM_CHANNEL_1);

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV2;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief TIM2 Initialization Function

* @param None

* @retval None

*/

static void MX_TIM2_Init(void)

{

/* USER CODE BEGIN TIM2_Init 0 */

/* USER CODE END TIM2_Init 0 */

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

/* USER CODE BEGIN TIM2_Init 1 */

/* USER CODE END TIM2_Init 1 */

htim2.Instance = TIM2;

htim2.Init.Prescaler = 127;

htim2.Init.CounterMode = TIM_COUNTERMODE_UP;

htim2.Init.Period = 62499;

htim2.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim2.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim2) != HAL_OK)

{

Error_Handler();

}

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim2, &sClockSourceConfig) != HAL_OK)

{

Error_Handler();

}

if (HAL_TIM_PWM_Init(&htim2) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim2, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 0;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

if (HAL_TIM_PWM_ConfigChannel(&htim2, &sConfigOC, TIM_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM2_Init 2 */

/* USER CODE END TIM2_Init 2 */

HAL_TIM_MspPostInit(&htim2);

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOA_CLK_ENABLE();

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/

- USART USING HAL_DRIVERS

#include<stdio.h>

#include<string.h>

#include "stm32f407xx.h"

char msg[1024] = "UART Tx testing...\n\r";

USART_Handle_t usart2_handle;

void USART2_Init(void)

{

usart2_handle.pUSARTx = USART2;

usart2_handle.USART_Config.USART_Baud = USART_STD_BAUD_115200;

usart2_handle.USART_Config.USART_HWFlowControl = USART_HW_FLOW_CTRL_NONE;

usart2_handle.USART_Config.USART_Mode = USART_MODE_ONLY_TX;

usart2_handle.USART_Config.USART_NoOfStopBits = USART_STOPBITS_1;

usart2_handle.USART_Config.USART_WordLength = USART_WORDLEN_8BITS;

usart2_handle.USART_Config.USART_ParityControl = USART_PARITY_DISABLE;

USART_Init(&usart2_handle);

}

void 	USART2_GPIOInit(void)

{

GPIO_Handle_t usart_gpios;

usart_gpios.pGPIOx = GPIOA;

usart_gpios.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_ALTFN;

usart_gpios.GPIO_PinConfig.GPIO_PinOPType = GPIO_OP_TYPE_PP;

usart_gpios.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_PIN_PU;

usart_gpios.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

usart_gpios.GPIO_PinConfig.GPIO_PinAltFunMode =7;

//USART2 TX

usart_gpios.GPIO_PinConfig.GPIO_PinNumber  = GPIO_PIN_NO_2;

GPIO_Init(&usart_gpios);

//USART2 RX

usart_gpios.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_3;

GPIO_Init(&usart_gpios);

}

void GPIO_ButtonInit(void)

{

GPIO_Handle_t GPIOBtn,GpioLed;

//this is btn gpio configuration

GPIOBtn.pGPIOx = GPIOA;

GPIOBtn.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_0;

GPIOBtn.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_IN;

GPIOBtn.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GPIOBtn.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

GPIO_Init(&GPIOBtn);

//this is led gpio configuration

GpioLed.pGPIOx = GPIOD;

GpioLed.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

GpioLed.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_OUT;

GpioLed.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

GpioLed.GPIO_PinConfig.GPIO_PinOPType = GPIO_OP_TYPE_OD;

GpioLed.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

GPIO_PeriClockControl(GPIOD,ENABLE);

GPIO_Init(&GpioLed);

}

void delay(void)

{

for(uint32_t i = 0 ; i < 500000/2 ; i ++);

}

int main(void)

{

GPIO_ButtonInit();

USART2_GPIOInit();

USART2_Init();

USART_PeripheralControl(USART2,ENABLE);

while(1)

{

//wait till button is pressed

while( ! GPIO_ReadFromInputPin(GPIOA,GPIO_PIN_NO_0) );

//to avoid button de-bouncing related issues 200ms of delay

delay();

USART_SendData(&usart2_handle,(uint8_t*)msg,strlen(msg));

}

return 0;

}

SERIAL PERIPHERAL INTERFACE(SPI):

The SPI interface provides two main functions, supporting either the SPI protocol or the I2Saudio protocol. By default, it is the SPI function that is selected. It is possible to switch the interface from SPI to I2S by software.

The serial peripheral interface (SPI) allows half/ full-duplex, synchronous, serial

communication with external devices. The interface can be configured as the master and in this case it provides the communication clock (SCK) to the external slave device. The

interface is also capable of operating in multi-master configuration.

It may be used for a variety of purposes, including simplex synchronous transfers on two

lines with a possible bidirectional data line or reliable communication using CRC checking.

The I2S is also a synchronous serial communication interface. It can address four different audio standards including the I2S Philips standard, the MSB- and LSB-justified standards, and the PCM standard. It can operate as a slave or a master device in full-duplex mode (using 4 pins) or in half-duplex mode (using 3 pins). Master clock can be provided by the interface to an external slave component when the I2S is configured as the communication

master.

SPI features

• Full-duplex synchronous transfers on three lines

• Simplex synchronous transfers on two lines with or without a bidirectional data line

• 8- or 16-bit transfer frame format selection

• Master or slave operation

• Multi-master mode capability

• 8 master mode baud rate pre-scalers (fPCLK/2 max.)

• Slave mode frequency (fPCLK/2 max)

• Faster communication for both master and slave

• NSS management by hardware or software for both master and slave: dynamic change

of master/slave operations

• Programmable clock polarity and phase

• Programmable data order with MSB-first or LSB-first shifting

• Dedicated transmission and reception flags with interrupt capability

• SPI bus busy status flag

• SPI TI mode

• Hardware CRC feature for reliable communication:

– CRC value can be transmitted as last byte in Tx mode

– Automatic CRC error checking for last received byte

• Master mode fault, overrun and CRC error flags with interrupt capability

• 1-byte transmission and reception buffer with DMA capability: Tx and Rx requests

Usually, the SPI is connected to external devices through four pins:

• MISO: Master In / Slave Out data. This pin can be used to transmit data in slave mode

and receive data in master mode.

• MOSI: Master Out / Slave In data. This pin can be used to transmit data in master

mode and receive data in slave mode.

• SCK: Serial Clock output for SPI masters and input for SPI slaves.

• NSS: Slave select. This is an optional pin to select a slave device. This pin acts as a

‘chip select’ to let the SPI master communicate with slaves individually and to avoid

contention on the data lines. Slave NSS inputs can be driven by standard IO ports on

the master device. The NSS pin may also be used as an output if enabled (SSOE bit)

and driven low if the SPI is in master configuration.

SPI DRIVERS SPECIFIC:

typedef struct

{

uint8_t SPI_DeviceMode;

uint8_t SPI_BusConfig;

uint8_t SPI_SclkSpeed;

uint8_t SPI_DFF;

uint8_t SPI_CPOL;

uint8_t SPI_CPHA;

uint8_t SPI_SSM;

}SPI_Config_t;

/*

*Handle structure for SPIx peripheral

*/

typedef struct

{

SPI_RegDef_t *pSPIx;   /*!< This holds the base address of SPIx(x:0,1,2) peripheral >*/

SPI_Config_t  SPIConfig;

uint8_t  *pTxBuffer; /* !< To store the app. Tx buffer address > */

uint8_t  *pRxBuffer;	/* !< To store the app. Rx buffer address > */

uint32_t  TxLen;		/* !< To store Tx len > */

uint32_t  RxLen;		/* !< To store Tx len > */

uint8_t  TxState;	/* !< To store Tx state > */

uint8_t  RxState;	/* !< To store Rx state > */

}SPI_Handle_t;

/*

* SPI application states

*/

#define SPI_READY 					0

#define SPI_BUSY_IN_RX 				1

#define SPI_BUSY_IN_TX 				2

/*

* Possible SPI Application events

*/

#define SPI_EVENT_TX_CMPLT   1

#define SPI_EVENT_RX_CMPLT   2

#define SPI_EVENT_OVR_ERR    3

#define SPI_EVENT_CRC_ERR    4

/*

* @SPI_DeviceMode

*/

#define SPI_DEVICE_MODE_MASTER    1

#define SPI_DEVICE_MODE_SLAVE     0

/*

* @SPI_BusConfig

*/

#define SPI_BUS_CONFIG_FD                1

#define SPI_BUS_CONFIG_HD                2

#define SPI_BUS_CONFIG_SIMPLEX_RXONLY    3

/*

* @SPI_SclkSpeed

*/

#define SPI_SCLK_SPEED_DIV2             	0

#define SPI_SCLK_SPEED_DIV4             	1

#define SPI_SCLK_SPEED_DIV8             	2

#define SPI_SCLK_SPEED_DIV16             	3

#define SPI_SCLK_SPEED_DIV32             	4

#define SPI_SCLK_SPEED_DIV64             	5

#define SPI_SCLK_SPEED_DIV128             	6

#define SPI_SCLK_SPEED_DIV256             	7

/*

* @SPI_DFF

*/

#define SPI_DFF_8BITS 	0

#define SPI_DFF_16BITS  1

/*

* @CPOL

*/

#define SPI_CPOL_HIGH 1

#define SPI_CPOL_LOW 0

/*

* @CPHA

*/

#define SPI_CPHA_HIGH 1

#define SPI_CPHA_LOW 0

/*

* @SPI_SSM

*/

#define SPI_SSM_EN     1

#define SPI_SSM_DI     0

/*

* SPI related status flags definitions

*/

#define SPI_TXE_FLAG    ( 1 << SPI_SR_TXE)

#define SPI_RXNE_FLAG   ( 1 << SPI_SR_RXNE)

#define SPI_BUSY_FLAG   ( 1 << SPI_SR_BSY)

/*

* Peripheral Clock setup

*/

void SPI_PeriClockControl(SPI_RegDef_t *pSPIx, uint8_t EnorDi);

/*

* Init and De-init

*/

void SPI_Init(SPI_Handle_t *pSPIHandle);

void SPI_DeInit(SPI_RegDef_t *pSPIx);

/*

* Data Send and Receive

*/

void SPI_SendData(SPI_RegDef_t *pSPIx,uint8_t *pTxBuffer, uint32_t Len);

void SPI_ReceiveData(SPI_RegDef_t *pSPIx, uint8_t *pRxBuffer, uint32_t Len);

uint8_t SPI_SendDataIT(SPI_Handle_t *pSPIHandle,uint8_t *pTxBuffer, uint32_t Len);

uint8_t SPI_ReceiveDataIT(SPI_Handle_t *pSPIHandle, uint8_t *pRxBuffer, uint32_t Len);

/*

* IRQ Configuration and ISR handling

*/

void SPI_IRQInterruptConfig(uint8_t IRQNumber, uint8_t EnorDi);

void SPI_IRQPriorityConfig(uint8_t IRQNumber, uint32_t IRQPriority);

void SPI_IRQHandling(SPI_Handle_t *pHandle);

/*

* Other Peripheral Control APIs

*/

void SPI_PeripheralControl(SPI_RegDef_t *pSPIx, uint8_t EnOrDi);

void SPI_SSIConfig(SPI_RegDef_t *pSPIx, uint8_t EnOrDi);

void SPI_SSOEConfig(SPI_RegDef_t *pSPIx, uint8_t EnOrDi);

uint8_t SPI_GetFlagStatus(SPI_RegDef_t *pSPIx , uint32_t FlagName);

void SPI_ClearOVRFlag(SPI_RegDef_t *pSPIx);

void SPI_CloseTransmisson(SPI_Handle_t *pSPIHandle);

void SPI_CloseReception(SPI_Handle_t *pSPIHandle);

uint8_t I2C_DeviceMode(I2C_RegDef_t *I2Cx);

/*

* Application callback

*/

void SPI_ApplicationEventCallback(SPI_Handle_t *pSPIHandle,uint8_t AppEv);

#endif /* INC_STM32F407XX_SPI_DRIVER_H_ */

DRIVERS MAIN FILE:

static void  spi_txe_interrupt_handle(SPI_Handle_t *pSPIHandle);

static void  spi_rxne_interrupt_handle(SPI_Handle_t *pSPIHandle);

static void  spi_ovr_err_interrupt_handle(SPI_Handle_t *pSPIHandle);

void SPI_PeriClockControl(SPI_RegDef_t *pSPIx, uint8_t EnorDi)

{

if(EnorDi == ENABLE)

{

if(pSPIx == SPI1)

{

SPI1_PCLK_EN();

}else if (pSPIx == SPI2)

{

SPI2_PCLK_EN();

}else if (pSPIx == SPI3)

{

SPI3_PCLK_EN();

}

}

else

{

//TODO

}

}

void SPI_Init(SPI_Handle_t *pSPIHandle)

{

//peripheral clock enable

SPI_PeriClockControl(pSPIHandle->pSPIx, ENABLE);

//first lets configure the SPI_CR1 register

uint32_t tempreg = 0;

//1. configure the device mode

tempreg |= pSPIHandle->SPIConfig.SPI_DeviceMode << SPI_CR1_MSTR ;

//2. Configure the bus config

if(pSPIHandle->SPIConfig.SPI_BusConfig == SPI_BUS_CONFIG_FD)

{

//bidi mode should be cleared

tempreg &= ~( 1 << SPI_CR1_BIDIMODE);

}else if (pSPIHandle->SPIConfig.SPI_BusConfig == SPI_BUS_CONFIG_HD)

{

//bidi mode should be set

tempreg |= ( 1 << SPI_CR1_BIDIMODE);

}else if (pSPIHandle->SPIConfig.SPI_BusConfig ==   SPI_BUS_CONFIG_SIMPLEX_RXONLY)

{

//BIDI mode should be cleared

tempreg &= ~( 1 << SPI_CR1_BIDIMODE);

//RXONLY bit must be set

tempreg |= ( 1 << SPI_CR1_RXONLY);

}

// 3. Configure the spi serial clock speed (baud rate)

tempreg |= pSPIHandle->SPIConfig.SPI_SclkSpeed << SPI_CR1_BR;

//4.  Configure the DFF

tempreg |= pSPIHandle->SPIConfig.SPI_DFF << SPI_CR1_DFF;

//5. configure the CPOL

tempreg |= pSPIHandle->SPIConfig.SPI_CPOL << SPI_CR1_CPOL;

//6 . configure the CPHA

tempreg |= pSPIHandle->SPIConfig.SPI_CPHA << SPI_CR1_CPHA;

tempreg |= pSPIHandle->SPIConfig.SPI_SSM << SPI_CR1_SSM;

pSPIHandle->pSPIx->CR1 = tempreg;

}

void SPI_DeInit(SPI_RegDef_t *pSPIx)

{

//todo

}

uint8_t SPI_GetFlagStatus(SPI_RegDef_t *pSPIx , uint32_t FlagName)

{

if(pSPIx->SR & FlagName)

{

return FLAG_SET;

}

return FLAG_RESET;

}

void SPI_SendData(SPI_RegDef_t *pSPIx,uint8_t *pTxBuffer, uint32_t Len)

{

while(Len > 0)

{

//1. wait until TXE is set

while(SPI_GetFlagStatus(pSPIx,SPI_TXE_FLAG)  == FLAG_RESET );

//2. check the DFF bit in CR1

if( (pSPIx->CR1 & ( 1 << SPI_CR1_DFF) ) )

{

//16 bit DFF

//1. load the data in to the DR

pSPIx->DR =   *((uint16_t*)pTxBuffer);

Len--;

Len--;

(uint16_t*)pTxBuffer++;

}else

{

//8 bit DFF

pSPIx->DR =   *pTxBuffer;

Len--;

pTxBuffer++;

}

}

}

void SPI_ReceiveData(SPI_RegDef_t *pSPIx, uint8_t *pRxBuffer, uint32_t Len)

{

while(Len > 0)

{

//1. wait until RXNE is set

while(SPI_GetFlagStatus(pSPIx,SPI_RXNE_FLAG)  ==  (uint8_t)FLAG_RESET );

//2. check the DFF bit in CR1

if( (pSPIx->CR1 & ( 1 << SPI_CR1_DFF) ) )

{

//16 bit DFF

//1. load the data from DR to Rxbuffer address

*((uint16_t*)pRxBuffer) = pSPIx->DR ;

Len--;

Len--;

(uint16_t*)pRxBuffer++;

}else

{

//8 bit DFF

*(pRxBuffer) = pSPIx->DR ;

Len--;

pRxBuffer++;

}

}

}

void SPI_PeripheralControl(SPI_RegDef_t *pSPIx, uint8_t EnOrDi)

{

if(EnOrDi == ENABLE)

{

pSPIx->CR1 |=  (1 << SPI_CR1_SPE);

}else

{

pSPIx->CR1 &=  ~(1 << SPI_CR1_SPE);

}

}

void  SPI_SSIConfig(SPI_RegDef_t *pSPIx, uint8_t EnOrDi)

{

if(EnOrDi == ENABLE)

{

pSPIx->CR1 |=  (1 << SPI_CR1_SSI);

}else

{

pSPIx->CR1 &=  ~(1 << SPI_CR1_SSI);

}

}

void  SPI_SSOEConfig(SPI_RegDef_t *pSPIx, uint8_t EnOrDi)

{

if(EnOrDi == ENABLE)

{

pSPIx->CR2 |=  (1 << SPI_CR2_SSOE);

}else

{

pSPIx->CR2 &=  ~(1 << SPI_CR2_SSOE);

}

}

void SPI_IRQInterruptConfig(uint8_t IRQNumber, uint8_t EnorDi)

{

if(EnorDi == ENABLE)

{

if(IRQNumber <= 31)

{

//program ISER0 register

*NVIC_ISER0 |= ( 1 << IRQNumber );

}else if(IRQNumber > 31 && IRQNumber < 64 ) //32 to 63

{

//program ISER1 register

*NVIC_ISER1 |= ( 1 << (IRQNumber % 32) );

}

else if(IRQNumber >= 64 && IRQNumber < 96 )

{

//program ISER2 register //64 to 95

*NVIC_ISER3 |= ( 1 << (IRQNumber % 64) );

}

}else

{

if(IRQNumber <= 31)

{

//program ICER0 register

*NVIC_ICER0 |= ( 1 << IRQNumber );

}else if(IRQNumber > 31 && IRQNumber < 64 )

{

//program ICER1 register

*NVIC_ICER1 |= ( 1 << (IRQNumber % 32) );

}

else if(IRQNumber >= 6 && IRQNumber < 96 )

{

//program ICER2 register

*NVIC_ICER3 |= ( 1 << (IRQNumber % 64) );

}

}

}

void SPI_IRQPriorityConfig(uint8_t IRQNumber,uint32_t IRQPriority)

{

//1. first lets find out the ipr register

uint8_t iprx = IRQNumber / 4;

uint8_t iprx_section  = IRQNumber %4 ;

uint8_t shift_amount = ( 8 * iprx_section) + ( 8 - NO_PR_BITS_IMPLEMENTED) ;

*(  NVIC_PR_BASE_ADDR + iprx ) |=  ( IRQPriority << shift_amount );

}

uint8_t SPI_SendDataIT(SPI_Handle_t *pSPIHandle,uint8_t *pTxBuffer, uint32_t Len)

{

uint8_t state = pSPIHandle->TxState;

if(state != SPI_BUSY_IN_TX)

{

//1 . Save the Tx buffer address and Len information in some global variables

pSPIHandle->pTxBuffer = pTxBuffer;

pSPIHandle->TxLen = Len;

//2.  Mark the SPI state as busy in transmission so that

//    no other code can take over same SPI peripheral until transmission is  over

pSPIHandle->TxState = SPI_BUSY_IN_TX;

//3. Enable the TXEIE control bit to get interrupt whenever TXE flag is set in SR

pSPIHandle->pSPIx->CR2 |= ( 1 << SPI_CR2_TXEIE );

}

return state;

}

uint8_t SPI_ReceiveDataIT(SPI_Handle_t *pSPIHandle, uint8_t *pRxBuffer, uint32_t Len)

{

uint8_t state = pSPIHandle->RxState;

if(state != SPI_BUSY_IN_RX)

{

//1 . Save the Rx buffer address and Len information in some global variables

pSPIHandle->pRxBuffer = pRxBuffer;

pSPIHandle->RxLen = Len;

//2.  Mark the SPI state as busy in reception so that

//    no other code can take over same SPI peripheral until reception is over

pSPIHandle->RxState = SPI_BUSY_IN_RX;

//3. Enable the RXNEIE control bit to get interrupt whenever RXNEIE flag is set in SR

pSPIHandle->pSPIx->CR2 |= ( 1 << SPI_CR2_RXNEIE );

}

return state;

}

void SPI_IRQHandling(SPI_Handle_t *pHandle)

{

uint8_t temp1 , temp2;

//first lets check for TXE

temp1 = pHandle->pSPIx->SR & ( 1 << SPI_SR_TXE);

temp2 = pHandle->pSPIx->CR2 & ( 1 << SPI_CR2_TXEIE);

if( temp1 && temp2)

{

//handle TXE

spi_txe_interrupt_handle(pHandle);

}

// check for RXNE

temp1 = pHandle->pSPIx->SR & ( 1 << SPI_SR_RXNE);

temp2 = pHandle->pSPIx->CR2 & ( 1 << SPI_CR2_RXNEIE);

if( temp1 && temp2)

{

//handle RXNE

spi_rxne_interrupt_handle(pHandle);

}

// check for ovr flag

temp1 = pHandle->pSPIx->SR & ( 1 << SPI_SR_OVR);

temp2 = pHandle->pSPIx->CR2 & ( 1 << SPI_CR2_ERRIE);

if( temp1 && temp2)

{

//handle ovr error

spi_ovr_err_interrupt_handle(pHandle);

}

}

//some helper function implementations

static void  spi_txe_interrupt_handle(SPI_Handle_t *pSPIHandle)

{

// check the DFF bit in CR1

if( (pSPIHandle->pSPIx->CR1 & ( 1 << SPI_CR1_DFF) ) )

{

//16 bit DFF

//1. load the data in to the DR

pSPIHandle->pSPIx->DR =   *((uint16_t*)pSPIHandle->pTxBuffer);

pSPIHandle->TxLen--;

pSPIHandle->TxLen--;

(uint16_t*)pSPIHandle->pTxBuffer++;

}else

{

//8 bit DFF

pSPIHandle->pSPIx->DR =   *pSPIHandle->pTxBuffer;

pSPIHandle->TxLen--;

pSPIHandle->pTxBuffer++;

}

if(! pSPIHandle->TxLen)

{

//TxLen is zero , so close the spi transmission and inform the application that

//TX is over.

//this prevents interrupts from setting up of TXE flag

SPI_CloseTransmisson(pSPIHandle);

SPI_ApplicationEventCallback(pSPIHandle,SPI_EVENT_TX_CMPLT);

}

}

static void  spi_rxne_interrupt_handle(SPI_Handle_t *pSPIHandle)

{

//do rxing as per the dff

if(pSPIHandle->pSPIx->CR1 & ( 1 << 11))

{

//16 bit

*((uint16_t*)pSPIHandle->pRxBuffer) = (uint16_t) pSPIHandle->pSPIx->DR;

pSPIHandle->RxLen -= 2;

pSPIHandle->pRxBuffer++;

pSPIHandle->pRxBuffer++;

}

else

{

//8 bit

*(pSPIHandle->pRxBuffer) = (uint8_t) pSPIHandle->pSPIx->DR;

pSPIHandle->RxLen--;

pSPIHandle->pRxBuffer++;

}

if(! pSPIHandle->RxLen)

{

//reception is complete

SPI_CloseReception(pSPIHandle);

SPI_ApplicationEventCallback(pSPIHandle,SPI_EVENT_RX_CMPLT);

}

}

static void  spi_ovr_err_interrupt_handle(SPI_Handle_t *pSPIHandle)

{

uint8_t temp;

//1. clear the ovr flag

if(pSPIHandle->TxState != SPI_BUSY_IN_TX)

{

temp = pSPIHandle->pSPIx->DR;

temp = pSPIHandle->pSPIx->SR;

}

(void)temp;

//2. inform the application

SPI_ApplicationEventCallback(pSPIHandle,SPI_EVENT_OVR_ERR);

}

void SPI_CloseTransmisson(SPI_Handle_t *pSPIHandle)

{

pSPIHandle->pSPIx->CR2 &= ~( 1 << SPI_CR2_TXEIE);

pSPIHandle->pTxBuffer = NULL;

pSPIHandle->TxLen = 0;

pSPIHandle->TxState = SPI_READY;

}

void SPI_CloseReception(SPI_Handle_t *pSPIHandle)

{

pSPIHandle->pSPIx->CR2 &= ~( 1 << SPI_CR2_RXNEIE);

pSPIHandle->pRxBuffer = NULL;

pSPIHandle->RxLen = 0;

pSPIHandle->RxState = SPI_READY;

}

void SPI_ClearOVRFlag(SPI_RegDef_t *pSPIx)

{

uint8_t temp;

temp = pSPIx->DR;

temp = pSPIx->SR;

(void)temp;

}

__weak void SPI_ApplicationEventCallback(SPI_Handle_t *pSPIHandle,uint8_t AppEv)

{

}

SPI_TESTING:

#include<string.h>

#include "stm32f407xx.h"

/*

* PB14 --> SPI2_MISO

* PB15 --> SPI2_MOSI

* PB13 -> SPI2_SCLK

* PB12 --> SPI2_NSS

* ALT function mode : 5

*/

void SPI2_GPIOInits(void)

{

GPIO_Handle_t SPIPins;

SPIPins.pGPIOx = GPIOB;

SPIPins.GPIO_PinConfig.GPIO_PinMode = GPIO_MODE_ALTFN;

SPIPins.GPIO_PinConfig.GPIO_PinAltFunMode = 5;

SPIPins.GPIO_PinConfig.GPIO_PinOPType = GPIO_OP_TYPE_PP;

SPIPins.GPIO_PinConfig.GPIO_PinPuPdControl = GPIO_NO_PUPD;

SPIPins.GPIO_PinConfig.GPIO_PinSpeed = GPIO_SPEED_FAST;

//SCLK

SPIPins.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_13;

GPIO_Init(&SPIPins);

//MOSI

SPIPins.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_15;

GPIO_Init(&SPIPins);

//MISO//

SPIPins.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_14;

GPIO_Init(&SPIPins);

//NSS//

SPIPins.GPIO_PinConfig.GPIO_PinNumber = GPIO_PIN_NO_12;

GPIO_Init(&SPIPins);

}

void SPI2_Inits(void)

{

SPI_Handle_t SPI2handle;

SPI2handle.pSPIx = SPI2;

SPI2handle.SPIConfig.SPI_BusConfig = SPI_BUS_CONFIG_FD;

SPI2handle.SPIConfig.SPI_DeviceMode = SPI_DEVICE_MODE_MASTER;

SPI2handle.SPIConfig.SPI_SclkSpeed = SPI_SCLK_SPEED_DIV2;

//generates   sclk of 8MHz

SPI2handle.SPIConfig.SPI_DFF = SPI_DFF_8BITS;

SPI2handle.SPIConfig.SPI_CPOL = SPI_CPOL_HIGH;

SPI2handle.SPIConfig.SPI_CPHA = SPI_CPHA_LOW;

SPI2handle.SPIConfig.SPI_SSM = SPI_SSM_EN; //software slave management enabled for NSS pin

SPI_Init(&SPI2handle);

}

int main(void)

{

char user_data[] = "Hello world";

//this function is used to initialize the GPIO pins to behave as SPI2 pins

SPI2_GPIOInits();

//This function is used to initialize the SPI2 peripheral parameters

SPI2_Inits();

//this makes NSS signal internally high and avoids MODF error

SPI_SSIConfig(SPI2,ENABLE);

//enable the SPI2 peripheral

SPI_PeripheralControl(SPI2,ENABLE);

//to send data

SPI_SendData(SPI2,(uint8_t*)user_data,strlen(user_data));

//lets confirm SPI is not busy

while( SPI_GetFlagStatus(SPI2,SPI_BUSY_FLAG) );

//Disable the SPI2 peripheral

SPI_PeripheralControl(SPI2,DISABLE);

while(1);

return 0;

}